Programming Assignment: Multi-issue Tomasulo Simulator

Akhil P

1. What I Added

I started from the provided single-issue Python Tomasulo simulator (main, issue, exe, mem, wb, commit, init, print_status). I kept the same module and pipeline-stage structure. The state handling inside was rewritten around one Machine object, so that several instructions per cycle and speculative execution are handled correctly.

- Issue width 1 to 4 (in order) – issue.py. Up to W instructions are fetched, decoded and renamed per cycle, in program order. The issue group stops at the first structural hazard (ROB, reservation station or load/store queue full) or after a predicted-taken branch. Source operands are read from the RAT before the destination is renamed, so instructions in the same group see each other correctly (RAW inside a group). The issue width is read from the input file (“Issue width = N”, default 1) or from the command-line option --issue-width.

- Multiple write-backs per cycle – wb.py. Up to cdb_width results (default = issue width) are broadcast per cycle, oldest first. Setting “CDB width = 1” in the input file gives the single-CDB machine.

- Multiple commits per cycle – commit.py. Up to commit_width instructions (default = issue width) commit per cycle, strictly in program order.

- Branch prediction – the original code only stalled fetch at a branch, so I added a 1-bit predictor with an 8-entry BTB indexed by the 3 least significant bits of the word address of the PC (the entry is tagged with the PC). Prediction is done in the ISSUE stage and the branch is resolved at the end of EX. On a misprediction detected in cycle n, the RAT is restored from the checkpoint taken at the branch, all younger instructions are removed from the ROB, reservation stations, load/store queue, functional units and CDB queue, and fetch restarts on the correct path in cycle n+2.

- Input and output as in the assignment. One input file holds the hardware table, ROB size, issue width, register and memory initial values, and the program. The output is the instruction status table (ISSUE, EX, MEM, WB, COMMIT), the register values (R0–R31, F0–F31) and the non-zero memory contents.

- I fixed problems in the old code that break with width greater than 1: hard-coded configuration, the PC.valid stall, branch offsets handled as bytes, special cases for Sd/commit, and no final register/memory dump.

- Verification. The script tests/run_tests.py compares the final registers, memory and instruction count against a simple in-order reference interpreter. It also checks pipeline invariants: stage ordering, at most W issues per cycle, at most commit-width commits per cycle, at most CDB-width broadcasts per cycle, in-order issue and commit, and that the memory port is never used by two accesses at once. This is done for 8 hand-written tests at 4 issue widths, for CDB/commit-width variants, and for 400 random programs. Result: all checks pass.

2. Machine Model (as Implemented)

- Pipeline stages are ISSUE, EX, MEM, WB and COMMIT. An instruction issued in cycle c can start EX in cycle c+1 at the earliest. A result broadcast in cycle t can be used by a consumer starting EX in cycle t+1. An instruction commits in the cycle after its WB at the earliest (a branch: after it is resolved).

- The integer adder is unpipelined. The FP adder and FP multiplier are pipelined. Loads and stores use a dedicated address adder and do not occupy the integer ALU. Branches use the integer ALU and its reservation stations. Integer R0 is hard-wired to 0.

- Load/store queue: entries are kept in program order. A load goes to memory when its address is known, all older stores have known addresses, and none of them matches. If the nearest older store with the same address has its data, the load gets the value by forwarding (1 cycle, memory port not used). Loads leave the queue when they have their data. Stores write memory only at commit, occupy the single non-pipelined memory port for the memory latency, and are dequeued at that point.

- Execution is out of order (oldest ready instruction first). Issue and commit are in order.

3. Results (Cycles / IPC for Different Issue Widths)

| test | what it tests | W=1 | W=2 | W=3 | W=4 |
| --- | --- | --- | --- | --- | --- |
| test1_loop | FP loop (Ld/Mult/Add/Sd, Bne, 3 iterations) | 57 / 0.44 | 52 / 0.48 | 51 / 0.49 | 51 / 0.49 |
| test2_independent | independent int + FP ops | 34 / 0.35 | 29 / 0.41 | 27 / 0.44 | 27 / 0.44 |
| test3_dependent | RAW chains, WAW/WAR via renaming | 46 / 0.24 | 43 / 0.26 | 42 / 0.26 | 42 / 0.26 |
| test4_memory | forwarding, disambiguation, dependent addresses | 52 / 0.23 | 55 / 0.22 | 55 / 0.22 | 55 / 0.22 |
| test5_branch_loop | array sum loop, 10 iterations | 55 / 0.75 | 51 / 0.80 | 50 / 0.82 | 49 / 0.84 |
| test6_branch_mix | Beq/Bne taken/not taken, wrong-path Sd/Ld | 23 / 0.39 | 20 / 0.45 | 19 / 0.47 | 18 / 0.50 |
| test7_alt_branch | alternating branch (1-bit predictor worst case) | 78 / 0.56 | 71 / 0.62 | 71 / 0.62 | 71 / 0.62 |
| test8_small_resources | ROB=4, 1 RS/unit, CDB=1, commit=1 | 37 / 0.24 | 36 / 0.25 | 36 / 0.25 | 36 / 0.25 |

The full per-instruction tables for widths 1, 2 and 4 are in the results folder of the repository.

Observations. Wider issue helps most when the work is independent (test2) or branchy (test6). It cannot help a dependence chain (test3), nor a loop bounded by the 15-cycle FP multiply and the single memory port (test1). In test4, W=2 is 3 cycles slower than W=1. With W=2 the store reaches commit, and leaves the load/store queue, before the younger load looks for a forwarding match, so the load has to wait for the memory port. This is a timing effect of memory-port contention, not a functional error. The results are identical and were verified.

4. What Works and Limitations

What works: everything listed in Section 1, verified by the tests described there.

Limitations and assumptions:

- Branch offsets are counted in instructions, following the assignment (PC+4+offset<<2). The old code.in used offset -12 with a byte-style conversion and was not a correct loop, so test1_loop uses -8. The example in the assignment sheet (Bne R2,R3,-3 with R2 not equal to R3) is an endless loop by construction. The simulator stops with an error after 200000 cycles.

- Wrong-path instructions are removed when the branch resolves. The fetch of the resolution cycle itself is not performed, so the “squashed” statistic counts only wrong-path instructions that were really in the ROB.

- Memory is byte-addressed with one value per address. Overlapping or misaligned accesses are not modelled, because addresses are compared for equality. A store to an illegal address on the correct path raises an error.

- CDB width and commit width default to the issue width. The assignment text mentions one CDB, so use “CDB width = 1” in the input file to model that.